SEMESTER END REGULAR/SUPPLEMENTARY EXAMINATIONS (AR21/23), April – 2026

| U.G. | Civil Engineering |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025 - 26 |  | Sem. | 4th |  |
| Course Code | 21CE405/23CE405 | Course Title |  |  |  |
|  |  | Transportation Engineering |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 70 (SEVENTY) |

SECTION-I

7 x 2 = 14 Marks

1.

| No. | Questions (a to g) | RBT Level | COs |
| --- | --- | --- | --- |
| a | List any four recommendations of Jayakar committee report for highway development in India. | Remember | 1 |
| b | List any four road user characteristics. | Remember | 1 |
| c | Write a mathematical expression for finding the stopping sight distance at grades. | Remember | 3 |
| d | What is the necessity of transition curve in the horizontal alignment? | Remember | 3 |
| e | Mention any two types of parking methods. | Remember | 4 |
| f | Enumerate the different methods of carrying out traffic volume studies. | Remember | 4 |
| g | Write different traffic maneuvers observed at an at-grade intersection. | Remember | 6 |

SECTION-II

4 x 14 = 56 Marks

| No. |  | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) |  | Explain the factors controlling in fixing the highway alignment. | Understand | 1 | 8M |
|  |  | (b) |  | Explain various surveys to carried out before planning a highway for a given area. | Understand | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 3 |  | (a) |  | Discuss design procedure of super elevation as per IRC guidelines. | Understand | 1 | 8M |
|  |  | (b) |  | Explain the features of various road patterns and provide neat sketches where required. | Understand | 2 | 6M |
| 4 |  | (a) |  | The speed of overtaking and overtaken vehicles are 80 and 60 kmph respectively. If the acceleration of overtaking vehicle is 0.99 m/sec2, calculate the safe passing sight distance for a two-way traffic. Assume suitable data. | Apply | 3 | 8M |
|  |  | (b) |  | Discuss the classification of highways as per the Indian Roads Congress (IRC). | Understand | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 5 |  | (a) | Calculate the stopping sight distance on a highway at a descending gradient of 2%, for a design speed of 80 kmph. Assume any missing data as per IRC recommendations. |  | Apply | 3 | 8M |
|  |  | (b) | Explain the steps involved in PPR/DPR preparation of a highway project |  | Understand | 2 | 6M |
| 6 |  | (a) | Explain the different methods of carrying out spot-speed studies, and O-D studies, with advantages and dis-advantages of each method. |  | Understand | 4 | 8M |
|  |  | (b) | A Fixed time 2-phase signal is to be provided at an intersection having North-South and East-West Road where only straight-ahead traffic is permitted. The design hour and the saturation flows for these arms given in the following table Parameter; North; South; East; West. Design hour flow (q) PCU/h; 800; 400; 750; 1000. Saturation flow (s) PCU/h; 2400; 2000; 3000; 3000. Calculate the optimum cycle time and green times for the minimum overall delay. Consider all red time as 8 seconds. The time lost per phase due to starting delays can be assumed to be 2 seconds. The value of the amber period is 2 seconds. Sketch the timing diagram for each phase. | Apply | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 7 |  | (a) |  | Write a short note on: i) Annual average daily traffic ii) Thirtieth highest hourly volume iii) Running speed iv) 85th Percentile speed v) Desire lines | Understand | 4 | 8M |
|  |  | (b) |  | The average normal flow of traffic on cross roads A and B during the design period are 400 and 250 pcu per hour and saturation flow on these roads are estimated to be 1250 and 1000 pcu per hour respectively. The all-red time required for pedestrian crossing is 12 sec. Design two phase signal by Webster’s method | Apply | 5 | 6M |
| 8 |  | (a) |  | With a neat sketch, enumerate all the conflicts in a four-legged intersection with two-way traffic and pedestrian movements on all legs. | Understand | 6 | 8M |
|  |  | (b) |  | Explain different vehicular and human characteristics influencing the traffic parameters. Also, differentiate between collision and condition diagram | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 9 | (a) |  |  | Explain rotary intersection capacity and explain what are the factors considered in design of a rotary intersection. | Understand | 6 | 8M |
|  | (b) |  |  | Explain the following terms in signal design: a) Cycle time b) Phase c) Lost time d) Saturation flow | Understand | 5 | 6M |

***********