SEMESTER END REGULAR/SUPPLEMENTARY EXAMINATIONS (AR21/23), April 2026

| U.G. | EEE |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-2026 |  | Sem. | 4th |  |
| Course Code | 21EE403/23EE403 | Course Title |  |  |  |
|  |  | Power Electronics |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 70 (SEVENTY) |

SECTION-I

7 x 2 = 14 Marks

1.

| No. | Questions (a to g) | RBT Level | COs |
| --- | --- | --- | --- |
| a | Define latching current and holding current of SCR. | Remember | 1 |
| b | List any four applications of power electronics. | Remember | 1 |
| c | Define firing angle and extinction angle. | Remember | 3 |
| d | How does source inductance affect converter output voltage? | Understand | 3 |
| e | What is phase control in AC voltage regulators? | Understand | 4 |
| f | A cycloconverter converts 50Hz to 25Hz. Determine frequency ratio and output period. | Understand | 4 |
| g | What is modulation index and why is it important in PWM? | Remember | 6 |

SECTION-II

4 x 14 = 56 Marks

| No. |  | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) |  | Explain static V–I characteristics of SCR with neat sketch. | Understand | 1 | 8M |
|  |  | (b) |  | Discuss various turn-on methods of SCR. | Understand | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 3 |  | (a) |  | Derive two-transistor analogy of SCR and explain regenerative action. | Understand | 1 | 8M |
|  |  | (b) |  | Explain SCR protection techniques with circuits. | Understand | 2 | 6M |
| 4 |  | (a) |  | Derive expression for average output voltage of single-phase full converter with RL load. | Understand | 3 | 8M |
|  |  | (b) |  | Explain class B and class C commutation. | Understand | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 5 |  | (a) | Explain operation of single-phase half controlled converter with waveform. |  | Understand | 3 | 8M |
|  |  | (b) | A half-wave converter feeds 10Ω load with 230V supply, α = 30°. Determine average current. |  | Apply | 2 | 6M |
| 6 |  | (a) | Explain operation of single-phase AC voltage controller with resistive load. |  | Understand | 4 | 8M |
|  |  | (b) | Explain the operation of buck converter with the circuit diagram. |  | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 7 |  | (a) |  | Describe working of cycloconverter with neat waveform. | Understand | 4 | 8M |
|  |  | (b) |  | Explain the control strategies of chopper. | Understand | 5 | 6M |
|  |  | (a) |  | Explain sinusoidal PWM technique and its advantages. | Understand | 6 | 8M |
|  |  | (b) |  | A boost converter has input 100V and duty cycle 0.5. Calculate output voltage. If the operating frequency is 1kHz, Calculate the turn on period. | Apply | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 9 | (a) |  |  | Analyze operation of single-phase full bridge inverter with waveform. | Understand | 6 | 8M |
|  | (b) |  |  | Explain operation of boost converter with circuit diagram. | Understand | 5 | 6M |

***********