SEMESTER END SUPPLEMENTARY EXAMINATIONS (AR19), April – 2026

| U.G. | EEE |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-26 |  | Sem. | 4th |  |
| Course Code | 19EE401 | Course Title |  |  |  |
|  |  | AC Machines |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 60 (SIXTY) |

SECTION-I

6 x 2 = 12 Marks

1.

| No. | Questions (a to f) | RBT Level | COs |
| --- | --- | --- | --- |
| a | Define slip of a 3-phase induction motor and write its formula. | Remember | 1 |
| b | State the condition for maximum torque in a 3-phase induction motor. | Remember | 1 |
| c | List the starting methods of a 3-phase squirrel cage induction motor. | Remember | 3 |
| d | State why a single-phase induction motor is not self-starting. | Remember | 3 |
| e | Define distribution factor and pitch factor for an alternator. | Remember | 4 |
| f | State why a synchronous motor is called a doubly excited machine. | Remember | 6 |

SECTION-II

4 x 12 = 48 Marks

| No. |  | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) |  | With a neat diagram, explain the construction of a 3-phase squirrel cage induction motor | Understand | 1 | 7M |
|  |  | (b) |  | Derive the torque equation of a 3-phase induction motor and write the expressions for starting torque and maximum torque. | Apply | 2 | 5M |
| OR |  |  |  |  |  |  |  |
| 3 |  | (a) |  | Describe the principle of operation of a 3-phase induction motor and derive expressions for rotor EMF and rotor current at standstill and during running. | Understand | 1 | 7M |
|  |  | (b) |  | A 3-phase, 400 V, 50 Hz, 4-pole induction motor has rotor resistance R₂ = 0.04 Ω/ph and standstill rotor reactance X₂ = 0.2 Ω/ph. Find: (i) slip at maximum torque, and (ii) ratio of starting torque to maximum torque. | Apply | 2 | 5M |
| 4 |  | (a) |  | Explain any two starting methods of a 3-phase induction motor with neat circuit diagram. | Understand | 3 | 7M |
|  |  | (b) |  | Explain double revolving field theory of a single-phase induction motor and draw its equivalent circuit. | Understand | 2 | 5M |
| OR |  |  |  |  |  |  |  |
| 5 |  | (a) | Explain pole changing and rotor resistance methods of speed control of a 3-phase induction motor with relevant diagrams |  | Understand | 3 | 7M |
|  |  | (b) | A 3-phase, 400 V, 6-pole, 50 Hz star-connected induction motor runs at 960 rpm on full load. Find: (i) slip, (ii) rotor frequency, and (iii) rotor speed with respect to stator field. |  | Apply | 2 | 5M |
| 6 |  | (a) | Derive the EMF equation of a 3-phase alternator and explain armature winding pitch and distribution factors. |  | Understand | 4 | 7M |
|  |  | (b) | Explain the synchronous impedance method (EMF method) of finding voltage regulation of an alternator. |  | Understand | 5 | 5M |
| OR |  |  |  |  |  |  |  |
| 7 |  | (a) |  | Explain armature reaction in a synchronous generator for unity, lagging, and leading power factor loads with phasor diagrams. | Understand | 4 | 7M |
|  |  | (b) |  | A 3-phase, 6600 V, star-connected alternator has synchronous impedance of (1 + j10) Ω/ph. At full-load 0.8 pf lagging, calculate the percentage voltage regulation. | Apply | 5 | 5M |
| 8 |  | (a) |  | Draw the phasor diagram of a synchronous motor and explain the variation of armature current and power factor with excitation (V-curves and inverted V-curves). | Understand | 6 | 7M |
|  |  | (b) |  | Explain the MMF method of voltage regulation of an alternator, draw the phasor diagram, and compare it with the EMF method. | Understand | 5 | 5M |
| OR |  |  |  |  |  |  |  |
| 9 | (a) |  |  | Explain hunting in synchronous motors, state its causes, effects, and how damper windings suppress it. | Understand | 6 | 7M |
|  | (b) |  |  | Explain the ZPF (Potier triangle) method of finding voltage regulation of an alternator. | Understand | 5 | 5M |

***********