SEMESTER END SUPPLEMENTARY EXAMINATIONS (AR19), April – 2026

| U.G. | EEE |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-26 |  | Sem. | 4th |  |
| Course Code | 19EE402 | Course Title |  |  |  |
|  |  | Electrical Circuits II |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 60 (SIXTY) |

SECTION-I

6 x 2 = 12 Marks

1.

| No. | Questions (a to f) | RBT Level | COs |
| --- | --- | --- | --- |
| a | What is meant by time constant? What is its significance? | Understand | 1 |
| b | Write how an Inductor and capacitor behaves at an instantaneous conditions. | Remember | 1 |
| c | Give the properties of Incidence matrix. | Understand | 2 |
| d | How a capacitor with initial voltage is represented in S domain? | Remember | 3 |
| e | Define two-port network | Remember | 4 |
| f | Write the properties of LC driving point immitance functions. | Understand | 5 |

SECTION-II 6

4 x 12 = 48 Marks

| No. |  | Questions (2 to 9) |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) | Derive the expressions for transient current and voltages in RL series circuit excited by DC source. | Understand | 1 | 7M |
|  |  | (b) | Assuming the switch initially has been open for a long time. (i)Obtain an expression for iw in the circuit of fig.1 which is valid for all t≥0. (ii)Calculate iw at t=0 and t=1.3ns. | Apply | 3 | 5M |
| OR |  |  |  |  |  |  |
| 3 |  | (a) | Derive the expression of transient current flow in RC circuit and obtain the characteristics behavior for unit step excitation under zero initial conditions. | Understand | 1 | 7M |
|  |  | (b) | For the circuit shown in fig.2, determine the total current delivered by the source when the switch is closed at t=0. | Apply | 3 | 5M |
| 4 |  | (a) | Find the Y-parameters for the network shown in below fig.3. | Analyze | 4 | 7M |
|  |  | (b) | Obtain the reciprocity and symmetry conditions for hybrid parameters. | Understand | 4 | 5M |
| OR |  |  |  |  |  |  |
| 5 |  | (a) | Explain about Series connection and Parallel connection of Two Two-port Networks. | Apply | 4 | 7M |
|  |  | (b) | Explain the ABCD parameters in terms of Y parameters. |  | 4 | 5M |
| 6 |  | (a) | Explain the procedure for obtaining network functions of Ladder network. | Remember | 5 | 7M |
|  |  | (b) | Draw the oriented graph of the network shown in Fig.4.Also obtain the Fundamental Cut-set Matrix. | Analyze | 2 | 5M |
| OR |  |  |  |  |  |  |
| 7 |  | (a) | For the given graph shown in Fig.4, draw the number of possible trees and also write down the basic tie-set matrix for any tree. | Analyze | 2 | 7M |
|  |  | (b) | Draw the dual network for the given network shown in Fig.9. |  | 5 | 5M |
| 8 |  | (a) | Check the positive realness of the following function s2+2s+4(s+3)(s+1) | Analyze | 6 | 7M |
|  |  | (b) | Find the second Foster form of RL network for the function. Ys=s2+8s+15s2+5s+4 | Apply | 6 | 5M |
| OR |  |  |  |  |  |  |
| 9 | (a) |  | Define positive real function and mention its properties. Also write the properties of RL and RC driving point functions. | Understand | 6 | 7M |
|  | (b) |  | Find the second Cauer form of the given function. Zs=(s+1)(s+3)s(s+2) | Apply | 5 | 5M |

***********