SEMESTER END REGULAR EXAMINATIONS (AR21/23), AUGUST – 2026

| P.G. | EEE-PID |  | Degree | Master of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-26 |  | Sem. | 2nd |  |
| Course Code | 21PID016 | Course Title |  |  |  |
|  |  | Power Electronic Control of AC Drives |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 70 (SEVENTY) |

SECTION-I

7 x 2 = 14 Marks

1.

| No. | Questions (a to g) | RBT Level | COs |
| --- | --- | --- | --- |
| a | Sketch the torque-speed curve of an induction motor. Identify regions of stable and unstable operation. | Understand | 1 |
| b | Explain scalar control of VSI-fed induction motor drives. Why is it called “open-loop volts/Hz control”? | Understand | 1 |
| c | What is the basic difference between Static Kramer and Static Scherbius drives? | Understand | 3 |
| d | Why is slip regulation important in induction motor speed control? | Understand | 3 |
| e | Differentiate between direct and indirect vector control. | Understand | 4 |
| f | State the principle behind decoupling torque and flux in vector control. | Remember | 4 |
| g | Differentiate between PMSM and BLDC motors. | Understand | 6 |

SECTION-II

4 x 14 = 56 Marks

| No. | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | (a) |  | Explain the condition for maximum torque (pull-out torque) and derive the slip at which it occurs. | Understand | 1 | 8M |
|  | (b) |  | Derive the relationship between stator voltage and frequency for maintaining constant flux in volts/Hz control. | Apply | 2 | 6M |
| OR |  |  |  |  |  |  |
| 3 | (a) |  | A 3-phase, 400 V, 50 Hz induction motor has the following parameters (per phase referred to stator): R1=0.5 Ω, X1=1.2 Ω, R2'=0.6 Ω, X2'=1.5 Ω, Xm=30 Ω. At slip s=0.04, calculate: - Rotor copper loss - Mechanical power developed - Efficiency of the motor | Apply | 1 | 8M |
|  | (b) |  | Compare current-controlled VSI drives with voltage-controlled VSI drives in terms of dynamic response and harmonic performance. | Understand | 2 | 6M |
| 4 | (a) |  | Draw and explain the block diagram of a CSI-fed induction motor drive. | Understand | 3 | 8M |
|  | (b) |  | Derive the condition for constant Torque operation in volts/Hz control of VSI-fed drives | Apply | 2 | 6M |
| OR |  |  |  |  |  |  |
| 5 | (a) | Discuss the operation of a Static Scherbius drive. Explain modes of operation and advantages |  | Understand | 3 | 8M |
|  | (b) | Explain efficiency optimization control using flux programming. How does it minimize losses? |  | Understand | 2 | 6M |
| 6 | (a) | Derive the equations for indirect vector control of induction motors. |  | Understand | 4 | 8M |
|  | (b) | Explain the self-control of synchronous motor drive with neat diagram. |  | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |
| 7 | (a) |  | Explain direct vector control of induction motor with neat block diagram. | Understand | 4 | 8M |
|  | (b) |  | Discuss power factor control in synchronous motor drives. How is it achieved. | Understand | 5 | 6M |
| 8 | (a) |  | Analyse the working of a current-controlled variable reluctance motor servo drive. | Understand | 6 | 8M |
|  | (b) |  | Analyze the advantages of current control in improving performance of VRM servo drives. | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |
| 9 | (a) |  | Explain control scheme for PMSM using vector control. | Understand | 6 | 8M |
|  | (b) |  | Compare sinusoidal type BLDC motor drives with trapezoidal type drives. | Understand | 5 | 6M |

***********