SEMESTER END REGULAR/SUPPLEMENTARY EXAMINATIONS (AR23), DEC. – 2025

| U.G. | Common to All |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-26 |  | Sem. | 1st |  |
| Course Code | 23BEX01 | Course Title |  |  |  |
|  |  | BASIC ELECTRICAL & ELECTRONICS ENGINEERING |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 70 (SEVENTY) |

SECTION-I

7 x 2 = 14 Marks

1.

| No. | Questions (a to g) | RBT Level | COs |
| --- | --- | --- | --- |
| a | What is the equivalent resistance for two resistors (R1 & R2) connected in series and parallel? | Remember | 1 |
| b | State Kirchhoff’s Current Law (KCL). | Remember | 1 |
| c | What is the basic operating principle of a Moving Iron (MI) instrument? | Remember | 3 |
| d | What is the main purpose of earthing in an electrical system? | Remember | 3 |
| e | What is the function of a Zener diode in a voltage regulator circuit? | Remember | 4 |
| f | Draw the characteristics of Zener diode. | Remember | 4 |
| g | Convert (342)10 to binary. | Remember | 6 |

SECTION-II

4 x 14 = 56 Marks

| No. |  | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) |  | Explain and derive the expressions for RMS value, average value, form factor, and peak factor of a sinusoidal AC voltage waveform. | Understand | 1 | 8M |
|  |  | (b) |  | Explain the construction and working principle of a DC Generator with the help of a neat diagram. | Understand | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 3 |  | (a) |  | Find the loop currents for the circuit shown below using mesh analysis. | Understand | 1 | 8M |
|  |  | (b) |  | Explain the working principle and operation of a single phase transformer with the help of a neat diagram. | Understand | 2 | 6M |
| 4 |  | (a) |  | Explain the construction and working principle of a Wheatstone Bridge with a neat circuit diagram. | Understand | 3 | 8M |
|  |  | (b) |  | Draw the layout of a hydroelectric power plant and explain the working of each component. | Understand | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 5 |  | (a) | Explain the construction and working principle of a PMMC instrument with a neat diagram. |  | Understand | 3 | 8M |
|  |  | (b) | Draw the layout of a solar power plant and explain the working of each component. |  | Understand | 2 | 6M |
| 6 |  | (a) | Explain the CB configuration of a Bipolar Junction Transistor (BJT) with the help of circuit diagrams and characteristics. |  | Understand | 4 | 8M |
|  |  | (b) | Explain the working of a full-wave bridge rectifier with the help of a neat circuit diagram and waveform. |  | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 7 |  | (a) |  | Explain the PN junction diode forward and reverse bias conditions with help of VI characteristics. | Understand | 4 | 8M |
|  |  | (b) |  | Explain the working of a half-wave bridge rectifier with the help of a neat circuit diagram and waveform. | Understand | 5 | 6M |
| 8 |  | (a) |  | Minimize the following using 3 variable k-map. F(A,B,C) = ∑ (1,3,5,7). | Understand | 6 | 8M |
|  |  | (b) |  | Explain the operation of DC power supply block diagram. | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 9 | (a) |  |  | Explain the truth tables of AND, OR, NAND and NOR gates. | Understand | 6 | 8M |
|  | (b) |  |  | Differentiate between half wave and full wave rectifiers. | Understand | 5 | 6M |

***********