SEMESTER END REGULAR/SUPPLEMENTARY EXAMINATIONS (AR23), DEC – 2025

| U.G. | Common to All |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-26 |  | Sem. | I |  |
| Course Code | 23CYX02 | Course Title |  |  |  |
|  |  | ENGINEERING CHEMISTRY |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 70 (SEVENTY) |

SECTION-I

7 x 2 = 14 Marks

1.

| No. | Questions (a to g) | RBT Level | COs |
| --- | --- | --- | --- |
| a | Define hardness of water. | Remember | 1 |
| b | What is priming and foaming? | Remember | 1 |
| c | Define corrosion and give one example. | Understand | 3 |
| d | Define Galvanic corrosion. | Remember | 3 |
| e | Calculate the Gross calorific value of fuel containing 85% of carbon, 3% of nitrogen, 2% of hydrogen, 0.5% of oxygen and 1% of sulphur. | Apply | 4 |
| f | Identify the monomeric units of Bakelite. | Understand | 4 |
| g | Define Flash point and Fire point. | Remember | 6 |

SECTION-II

4 x 14 = 56 Marks

| No. |  | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) |  | Calculate the total hardness, non-carbonate hardness (permanent) and carbonate hardness (Temporary) of a water sample containing Mg(HCO3)2 = 16.0 mg/L; Ca(HCO3)2 =16.2 mg/L, CaCl2 = 11.1 mg/L, CaSO4 = 13.6 mg/L, and MgSO4 = 12 mg/L. | Apply | 1 | 8M |
|  |  | (b) |  | Illustrate the construction, working and applications of a H2-O2 Fuel cell with necessary chemical reactions. | Understand | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 3 |  | (a) |  | Detail the operational principle of reverse osmosis and explain water purification by it. | Understand | 1 | 8M |
|  |  | (b) |  | Explain the construction and working of a Lithium ion battery with necessary chemical reactions in charging and discharging . | Understand | 2 | 6M |
| 4 |  | (a) |  | Explain the mechanism of wet corrosion. | Understand | 3 | 8M |
|  |  | (b) |  | Consider the cell Fe¦Fe2+(0.01M)¦¦Cu2+(0.5M)¦Cu. The standard electrode reduction potential of iron and copper are -0.44V and 0.34V respectively. Write the cell reaction and calculate the cell potential. | Apply | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 5 |  | (a) | Define electro plating. Explain the process of electro plating of copper. |  | Understand | 3 | 8M |
|  |  | (b) | Explain the construction, working and applications of a Zn-air battery. |  | Understand | 2 | 6M |
| 6 |  | (a) | Explain the proximate analysis of coal and its significance. |  | Understand | 4 | 8M |
|  |  | (b) | Explain the Bredig's Arc Method for the preparation of colloidal solutions of metals. Write any two advantages. |  | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 7 |  | (a) |  | Explain the preparation, properties and applications of Buna-S and PVC. | Understand | 4 | 8M |
|  |  | (b) |  | Illustrate any four differences between physical adsorption and chemical adsorption. | Understand | 5 | 6M |
| 8 |  | (a) |  | Explain the thick film lubrication mechanism in machines. | Understand | 6 | 8M |
|  |  | (b) |  | Describe the chemical method for synthesizing zinc oxide (ZnO) nanoparticles. | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 9 | (a) |  |  | Explain the manufacturing of Portland cement. | Understand | 6 | 8M |
|  | (b) |  |  | Discuss the applications of nanomaterials in the field of medicine and sensors. | Understand | 5 | 6M |

***********