SEMESTER END SUPPLEMENTARY EXAMINATIONS (AR20), FEB. – 2026

| U.G. | ECE |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-2026 |  | Sem. | III |  |
| Course Code | 20EC301 | Course Title |  |  |  |
|  |  | Electronic Devices and Circuits |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 60 (Sixty) |

SECTION-I

6 x 2 = 12 Marks

1.

| No. | Questions (a to f) | RBT Level | COs |
| --- | --- | --- | --- |
| a | Define forward static and dynamic resistances of diode. | Remember | 1 |
| b | Reproduce the equivalent circuit for tunnel diode. | Remember | 1 |
| c | Define Early Effect. | Remember | 3 |
| d | List any four advantages of FET over BJT. | Remember | 3 |
| e | What are the different types of FETs? | Remember | 4 |
| f | “n” number of stages are connected in cascade. Reproduce the expressions for the overall lower and higher cut-off frequencies. | Remember | 4 |

SECTION-II

4 x 12 = 48 Marks

| No. |  | Questions (2 to 9) |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) | Explain the operation of forward biased and reverse biased PN junction Diode and Draw the V-I characteristics of PN diode. | Understand | 1 | 7M |
|  |  | (b) | Zener diode can be used as a voltage regulator. Justify it. | Understand | 2 | 5M |
| OR |  |  |  |  |  |  |
| 3 |  | (a) | An AC supply of 220V is applied to a half wave rectifier circuit through a transformer with a turns ratio of 10:1. Assume the ideal diode. Find: i) DC output current ii) DC output voltage iii) Efficiency iv) Ripple Factor v) PIV. | Apply | 1 | 7M |
|  |  | (b) | Obtain the static and dynamic resistances of the PN junction germanium diode, if the temperature is 270C and Io=1μA for an applied forward bias voltage of 0.2 V. Assume =1.38×10-23 J/ok. | Apply | 2 | 5M |
| 4 |  | (a) | Explain the working of a N channel JFET and draw the V-I characteristics. | Understand | 3 | 7M |
|  |  | (b) | Draw and explain the circuit for bias compensation using diode. | Understand | 2 | 5M |
| OR |  |  |  |  |  |  |
| 5 |  | (a) | With neat diagram, explain the input and output characteristics of a transistor in CE configuration. | Understand | 3 | 7M |
|  |  | (b) | Explain the collector to base bias method along with circuit diagram and derive the stability factor for it. | Understand | 2 | 5M |
| 6 |  | (a) | Draw the CE amplifier with bypassed emitter resistance and derive the expression for its Ri and Av. | Understand | 4 | 7M |
|  |  | (b) | Sketch the circuit of CS amplifier and derive the expression for voltage gain at low frequencies. | Understand | 5 | 5M |
| OR |  |  |  |  |  |  |
| 7 |  | (a) | The h-parameters of CE-amplifier are hie = 1100Ω, hre = 2.5 × 10−4, hfe = 50, hoe = 24 µA/V and RS = 1 kΩ, RL = 10 kΩ. Determine current gain, voltage gain input and output impedances without taking source resistance into account. | Apply | 4 | 7M |
|  |  | (b) | Draw the frequency response of an amplifier and explain the effects of coupling and bypass capacitors. | Understand | 5 | 5M |
| 8 |  | (a) | Find the voltage gain and input resistances of a emitter follower at high frequencies. | Understand | 6 | 7M |
|  |  | (b) | Explain the operation of Darlington amplifier. | Understand | 5 | 5M |
| OR |  |  |  |  |  |  |
| 9 | (a) |  | Explain various high frequency parameters of a BJT. | Understand | 6 | 7M |
|  | (b) |  | State and explain Millers theorem. | Understand | 5 | 5M |

***********