SEMESTER END REGULAR/SUPPLEMENTARY EXAMINATIONS (AR23), JUNE– 2026

| U.G. | Common to all (N&O Sections) |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-26 |  | Sem. | 2nd |  |
| Course Code | 23BEX04 | Course Title |  |  |  |
|  |  | ENGINEERING GRAPHICS |  |  |  |
| Duration | 90 Min. | Maximum Marks |  |  | 35 (Thirty Five) |

SECTION-I

Conventional Drawing

| No | Questions | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- |
| 1 | The distance of the fixed point from the fixed line is equal to 50 mm and eccentricity is equal to 2/3. Construct an ellipse and draw normal and tangent to the curve at any point on it. | Apply | 1 | 7M |
| OR |  |  |  |  |
| 2 | An area of 144 sq. cm on a map represents an area of 36 sq. km on the field. Find the RF of the scale of the map and draw a diagonal scale to show km, hectometres and decametres and to measure up to 10 km. Indicate on the scale a distance 7 Km, 5 hectometres and 6 decimetres. | Apply | 1 | 7M |
| 3 | A line CD 80mm long is inclined at an angle of 300 to H.P and 450 to V.P. The point C is 20mm above H.P and 30mm in front of V.P. Draw the projections of the straight line. | Apply | 2 | 14M |
| OR |  |  |  |  |
| 4 | Draw the projections of a regular hexagon of 30mm side with its surface making an angle of 450 with HP. One of the sides of the hexagon is parallel and on the HP. | Apply | 2 | 14M |
| 5 | A regular pentagon lamina of 30 mm side, its surface is inclined at 300 to VP and side on which it rests on V.P makes an angle of 450 to H.P. Draw its projections. | Apply | 3 | 14M |
| OR |  |  |  |  |
| 6 | A cone of base 60 mm diameter and the axis 80 mm long lies on HP with its axis inclined at 450 to HP. Draw the top and front views of the cone. | Apply | 3 | 14M |

***********

SEMESTER END REGULAR/SUPPLEMENTARY EXAMINATIONS (AR23), JUNE– 2026

| U.G. | Common to all (N&O sections) |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-26 |  | Sem. | 2nd |  |
| Course Code | 23BEX04 | Course Title |  |  |  |
|  |  | ENGINEERING GRAPHICS |  |  |  |
| Duration | 90 Min. | Maximum Marks |  |  | 35 (Thirty Five) |

SECTION-II

Auto CAD Drawing

| No | Questions | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- |
| 7 | A hexagonal pyramid, having base with a 30 mm side and a 75 mm long axis, has one of its base edge is on the H.P., and vertical plane containing this edge and the axis is inclined at 300 to the V.P. Draw its projections when apex is 15 mm in front of the V.P. | Apply | 4 | 7M |
| OR |  |  |  |  |
| 8 | A hexagonal prism, edge of base 20 mm and axis 50 mm long, rests with its base on HP such that one of its rectangular faces is parallel to VP. It is cut by a plane perpendicular to VP, inclined at 45° to HP and passing through the right corner of the top face of the prism. Draw the projections and sectional top view. | Apply | 4 | 7M |
| 9 | Draw the development of a square prism of side of base 30mm and height 8Omm. | Apply | 5 | 14M |
| OR |  |  |  |  |
| 10 | A cylinder of diameter 40 mm and height 50 mm is resting vertically on one of its ends on the HP. It is cut by a plane perpendicular to the VP and inclined at 30° to the HP. The plane meets the axis at a point 30 mm from the base. Draw the development of the lateral surface of the lower portion of the truncated cylinder. | Apply | 5 | 14M |
| 11 | A hexagonal prism of base of side 30mm and height 60mm is resting on its base on H.P. Draw the isometric drawing of the prism. | Apply | 6 | 14M |
| OR |  |  |  |  |
| 12 | Draw the front view, top view and side view of the object shown in figure. All dimensions are in mm. Fig. 2 | Apply | 6 | 14M |

***********