SEMESTER END SUPPLEMENTARY EXAMINATIONS (AR21), MARCH-2026

| U.G. | Common to all |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-26 |  | Sem. | I |  |
| Course Code | 21CYX01 | Course Title |  |  |  |
|  |  | Engineering Chemistry |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 70 (SEVENTY) |

SECTION-I

7 x 2 = 14 Marks

1.

| No. | Questions (a to g) | RBT Level | COs |
| --- | --- | --- | --- |
| a | Define the functionality of a monomer. | Remember | 1 |
| b | Explain why vulcanized rubber shows better heat resistance than natural rubber. | Remember | 1 |
| c | Define corrosion with an example | Remember | 3 |
| d | Discuss the significance of the Pilling–Bedworth rule in corrosion. | Remember | 3 |
| e | Define HCV and LCV. | Remember | 4 |
| f | Define Octane Number. | Remember | 4 |
| g | Define Bond order. | Remember | 6 |

SECTION-II

4 x 14 = 56 Marks

| No. |  | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) |  | Explain the step growth polymerisation mechanism with an example. | Understand | 1 | 8M |
|  |  | (b) |  | Describe the construction, working and applications of Lead-acid battery. | Understand | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 3 |  | (a) |  | Illustrate the mechanism of conduction in polyacetylene and discuss the applications of conducting polymers. | Understand | 1 | 8M |
|  |  | (b) |  | Derive Nernst equation for electrode potential. | Understand | 2 | 6M |
| 4 |  | (a) |  | Explain differential aeration corrosion and galvanic corrosion with suitable illustration. | Remember | 3 | 8M |
|  |  | (b) |  | Find the emf of the cell with the following equation at 298 K Zn (S) + Cu2+ (0.001M) Zn2+ (0.001M) + Cu (S) (Eº =Zn2+/Zn = - 0.76 V, Eº=Cu2+/Cu = 0.34 V) | Apply | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 5 |  | (a) | Explain how corrosion is controlled by the impressed current cathodic technique. |  | Understand | 3 | 8M |
|  |  | (b) | Explain the construction and working of lithium ion battery |  | Understand | 2 | 6M |
| 6 |  | (a) | Discuss the determination of calorific value of a liquid fuel using a bomb calorimeter with a neat diagram. |  | Understand | 4 | 8M |
|  |  | (b) | Explain the important renewable energy source with examples. |  | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 7 |  | (a) |  | What is the composition of crude oil? Discuss steps involved in refining of crude petroleum. | Understand | 4 | 8M |
|  |  | (b) |  | Explain the principle, construction and working of Photo-voltaic cell. | Understand | 5 | 6M |
| 8 |  | (a) |  | Discuss the crystal field splitting of d-orbitals in case of octahedral complexes. | Understand | 6 | 8M |
|  |  | (b) |  | Explain the process of bio-diesel from any waste oil. | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 9 | (a) |  |  | Draw the π-molecular orbitals of benzene and discuss the presence of nodes | Understand | 6 | 8M |
|  | (b) |  |  | Explain the merits and demerits of solar energy in the generation of electrical energy. | Understand | 5 | 6M |

***********