SEMESTER END SUPPLEMENTARY EXAMINATIONS (AR23), MARCH –2026

| U.G. | Common to all |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-26 |  | Sem. | I |  |
| Course Code | 23CYX02 | Course Title |  |  |  |
|  |  | Engineering Chemistry |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 70 (SEVENTY) |

SECTION-I

7 x 2 = 14 Marks

1.

| No. | Questions (a to g) | RBT Level | COs |
| --- | --- | --- | --- |
| a | Define reverse osmosis | Remember | 1 |
| b | What is priming and foaming? | Remember | 1 |
| c | Define pilling Bedworth rule. | Remember | 3 |
| d | Define corrosion. Give an example | Remember | 3 |
| e | Define higher calorific value and lower calorific value | Remember | 4 |
| f | What is polymerization? Give any two natural polymers | Remember | 4 |
| g | What is refractory material? Give two examples. | Remember | 6 |

SECTION-II

4 x 14 = 56 Marks

| No. |  | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) |  | Define Hardness. Explain briefly about the temporary and permanent hardness of water with suitable examples. | Remember | 1 | 8M |
|  |  | (b) |  | Explain the construction, working and uses of H2-O2 Fuel cell | Understand | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 3 |  | (a) |  | How do you differentiate scale and a sludge formation from boiler water? Describe any four internal treatment methods of scale. | Understand | 1 | 8M |
|  |  | (b) |  | Explain the construction, working and uses of Ni-Cd cell | Understand | 2 | 6M |
| 4 |  | (a) |  | Explain briefly about the sacrificial anodic protection method with suitable example. | Understand | 3 | 8M |
|  |  | (b) |  | Illustrate the construction, working and uses of Zn-air battery | Understand | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 5 |  | (a) | Differentiate Galvanic corrosion and Differential aeration corrosion with suitable examples. |  | Analyze | 3 | 8M |
|  |  | (b) | Discuss about the working of Lithium ion batteries along with their anodic and cathodic reactions involved in it. |  | Understand | 2 | 6M |
| 6 |  | (a) | What is Bakelite? How Bakelite is formed? Give any two important applications of Bakelite. |  | Understand | 4 | 8M |
|  |  | (b) | Explain briefly about nanomaterials. Discuss chemical precipitation method for the preparation of nanomaterials. |  | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 7 |  | (a) |  | Discuss the analysis of coal using proximate analysis | Understand | 4 | 8M |
|  |  | (b) |  | Differentiate between physical and chemical adsorption | Analyze | 5 | 6M |
| 8 |  | (a) |  | What is lubricant. Discuss about Thick film lubrication mechanism. | Understand | 6 | 8M |
|  |  | (b) |  | Explain Bredig’s arc method for the preparation of colloidal solution | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 9 | (a) |  |  | Explain the manufacture of Portland cement with a neat sketch | Understand | 6 | 8M |
|  | (b) |  |  | Discuss biological synthesis of any nanomaterial. | Remember | 5 | 6M |

***********