SEMESTER END REGULAR EXAMINATIONS (AR23), MAY – 2026

| U.G. | CE |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2025-2026 |  | Sem. | 6th |  |
| Course Code | 23CEH12 | Course Title |  |  |  |
|  |  | Advanced Structural Analysis |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 70 (SEVENTY) |

SECTION-I

7 x 2 = 14 Marks

| 1. No. | Questions (a to g) | RBT Level | COs |
| --- | --- | --- | --- |
| a | Define the term RLD. | Remember | 1 |
| b | Define the term ILD. | Remember | 1 |
| c | State Upper bound theorem. | Remember | 3 |
| d | Define the term Plastic section modulus. | Remember | 3 |
| e | Define the term flexibility. | Remember | 4 |
| f | Define the term stiffness. | Remember | 4 |
| g | Write down the relationship between flexibility and stiffness. | Remember | 6 |

SECTION-II

4 x 14 = 56 Marks

| No. |  | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) |  | A simply supported beam of span 6 m carries a moving point load of 30 kN. Using influence line diagram, determine the maximum bending moment at a section located 2 m from the left support. | Apply | 1 | 8M |
|  |  | (b) |  | Determine the shape factor for a triangular section. | Apply | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 3 |  | (a) |  | A simply supported beam of span 12 m carries a uniformly distributed load of 5 kN/m over the entire span. Using influence line diagram, determine the maximum bending moment at a section located 4 m from the left support. | Apply | 1 | 8M |
|  |  | (b) |  | Determine the shape factor for a circular section. | Apply | 2 | 6M |
| 4 |  | (a) |  | Find out the plastic moment capacity of a fixed beam subjected to point load at its mid span. | Apply | 3 | 8M |
|  |  | (b) |  | Determine the shape factor for a rectangular section. | Apply | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 5 |  | (a) | Find out the plastic moment capacity of a simply supported beam subjected to uniformly distributed load w kN/m whole over the span. |  | Apply | 3 | 8M |
|  |  | (b) | Determine the shape factor for a T section having 120 mm X 10 mm web and 10 X 120 mm Flange dimension. |  | Apply | 2 | 6M |
| 6 |  | (a) | Find out the deflection at point B and C due to the applied external load for the given figure1. FigureF Figure.1. |  | Apply | 4 | 8M |
|  |  | (b) | Develop the stiffness matrix equation for Figure1. |  | Apply | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 7 |  | (a) |  | Find out the flexibility coefficient for the given Figure1. | Apply | 4 | 8M |
|  |  | (b) |  | Find out the final end moment and draw the end moment diagram for Figure1. | Apply | 5 | 6M |
| 8 |  | (a) |  | A continuous beam ABC with spans 5 m each carries no external load. The middle support B undergoes a settlement of 8 mm downward. Using the flexibility method, develop the flexibility matrix. | Apply | 6 | 8M |
|  |  | (b) |  | Find out the final end moment and draw the end moment diagram for Question No. 8(a). | Apply | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 9 | (a) |  |  | Develop the stiffness matrix equation for Figure1. if support B sinks 5 mm. | Apply | 6 | 8M |
|  | (b) |  |  | Differentiate flexibility and stiffness matrix methods. | Understand | 5 | 6M |

***********