SEMESTER END SUPPLEMENTARY EXAMINATIONS (AR20), May – 2026

| U.G. | ECE |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2026-27 |  | Sem. | 8th |  |
| Course Code | 20EC009 | Course Title |  |  |  |
|  |  | UHF and EHF Communication Systems |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 60 (SIXTY) |

SECTION-I

6x 2 = 12 Marks

1.

| No. | Questions (a to f) | RBT Level | COs |
| --- | --- | --- | --- |
| a | Define Kepler’s third law. | Remember | 1 |
| b | Define EIRP | Remember | 3 |
| c | Define noise temperature | Remember | 3 |
| d | List any two limitations of FMCW Radar | Remember | 4 |
| e | Define maximum unambiguous range | Remember | 4 |
| f | Define Radomes. | Remember | 6 |

SECTION-II

4 x 12 =48 Marks

| No. |  | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) |  | Explain about 14/11GHz bent pipe transponder in communication subsystem | Understand | 1 | 7M |
|  |  | (b) |  | Assess FDMA frame structure in detail. | Apply | 2 | 5M |
| OR |  |  |  |  |  |  |  |
| 3 |  | (a) |  | Explain telemetry, tracking, command, and monitoring subsystem of the satellite | Understand | 1 | 7M |
|  |  | (b) |  | Assess TDMA frame structure in detail. | Apply | 2 | 5M |
| 4 |  | (a) |  | Assess the power received by an earth station from a satellite transmitter based on flux density and its link budget equation. | Apply | 3 | 7M |
|  |  | (b) |  | GSM System uses a TDMA / FDD system. The GSM System uses a frame structure where each frame consist of 8 time slots, and each time slot contains 156.25 bits, and data is transmitted at 270.833 kbps in the channel. Find - Time duration of a bit - Time duration of a slot - Time duration of a frame How long must a user occupying a single slot must wait between two simultaneous transmissions? | Apply | 2 | 5M |
| OR |  |  |  |  |  |  |  |
| 5 |  | (a) | A satellite at 40,000 km from a point on the earth’s surface radiates a power of 10 W from an antenna with a gain of 17 dB in the direction of the observer. Find the flux density at the receiving point and the power received by an antenna at this point with an effective area of 10m2. |  | Apply | 3 | 7M |
|  |  | (b) | Explain the spreading and de-spreading PN sequence in CDMA with an example. |  | Understand | 2 | 5M |
| 6 |  | (a) | Assess the Radar block diagram with its operation. |  | Apply | 4 | 7M |
|  |  | (b) | Explain block diagram of FMCW radar with its advantages. |  | Understand | 5 | 5M |
| OR |  |  |  |  |  |  |  |
| 7 |  | (a) |  | Calculate the maximum range of Radar for the following specifications: - Peak power transmitted by the Radar, 𝑃𝑡 = 250𝐾𝑊 - Gain of transmitting Antenna, 𝐺 = 4000 - Effective aperture of the receiving Antenna, 𝐴𝑒 = 4 𝑚2 - Radar cross section of the target, 𝜎 = 25 𝑚2 Power of minimum detectable signal, 𝑆𝑚𝑖𝑛 = 10−12w | Apply | 4 | 7M |
|  |  | (b) |  | Explain the function of delay line canceller and derive an expression for the frequency response function | Understand | 5 | 5M |
| 8 |  | (a) |  | Explain Amplitude Comparison monopulse radar using one coordinate system. | Understand | 6 | 7M |
|  |  | (b) |  | Explain block diagram of CW radar with its advantages. | Understand | 5 | 5M |
| OR |  |  |  |  |  |  |  |
| 9 | (a) |  |  | Illustrate the operation of balanced type duplexer with neat sketch | Understand | 6 | 7M |
|  | (b) |  |  | Compare MTI versus Pulse Doppler Radar. | Understand | 5 | 5M |

***********